C-code Generator For RST controller Documentation

Table of Contents

1.	Abstract	2

2.	Problem statement	2

3.	Solution design	3

Gui Design	4

Calculation engine design	5

File Generator	5

Simulink model Generator	5

C-code Generator	5

Ide code generator(for testing)	6

4.	Validation Method	7

5.	Results (validation, Testing)	8

6.	Conclusion	19

7.	Video link on YouTube	19

8.	Appendix	19

9.	Contribution	27

10.	One Drive Link	27

11.	Team members	27

# Abstract

Objective : developing a configurable and buildable C-code library for generic RST digital controller. Simulation of closed loop control should be done on a simulator or evaluation board (e.g. TivaC EVB) for Arm Cortex M4 platform. Project ends at real time simulation or/and emulation of process and controller in closed loop. We reached the requirements using MATLAB (Gui  ,Simulink ,scripts ,code generation) ,keil, code blocks ,excel. We acquired  the definition files for the system ,and  generated       c code for the controller , validated and tested using a lecture example.

# Problem statement

The problem is when we need to control a plant with a specific regulation performance and characteristics , we have to calculate the RST parameters using matrices to solve Bezout equation, which requires several calculations and it is an inefficient use of time.

# Solution design

In this phase, we take the inputs form the user using the Gui , use them to calculate the RST parameter using the calculation engine  , generate the definitions files(confirmation .h, configuration .c ,RST Parameters .h , RST Parameters .c) using file Generator ,then we build a Simulink model with our Design to validate our calculations ,and check if that model is the required one ,then we generate the C-code for the model using the C-code generator after configuring the parameters of The Code generator ,we  take the file generated on the ide to build and test the signals to compare with our design on Simulink to validate and test the generated C-code before uploading it on our hardware (the c -code generated from MATLAB is totally hardware independent but we selected a specific hardware to fit our data type with the generated code).

## Gui Design

Using The MATLAB script that developed by ENG. Ayman ,which take the configuration as input and calculate the RST parameters and visualize the control signal and output signal ,we Developed much more Features as we would show in this Report.

- Inputs

- Plant Configuration

- Continues:  Ts, Delay ,plant num ,pant denominator.

- Discreate : B(z) and A(z).

- Regulation Performance .

- Load Disturbance  Frequency.

- Desired Tracking (Omega And Zeta).

- Outputs

  - Button generate definition files generate :

    - configuration files (input) (configuration .h , configuration .c ).

    - RST Parameters files (out of the calculation engine )( RST Parameters .h , RST Parameters .c).

  - Generate Automatic  C code Button :

    - Configure and Generate automatically the controller as a C-code file.

    - Generate Simulink model for the whole system to validate the requirement to the system .

    - Generate Simulink model for the controller to make sure the block has the features needed in the controller (filters, buffers).

  - Generate Simulation  C code Button :

    - Generate separate file for validation and testing the signals (U(k) ,E(k) ,Y(k)) ,that code to simulate the closed loop RST  parameters on   the system.

## Calculation engine design

By applying Bezout equation on the Regulation and the plant parameters  we get the Required R, S and T parameter.

pz-1=Az-1 Hsz-1 Sz-1+z-dBz-1 Hrz-1 Rz-1.

## File Generator

In this stage , we generate four files ,two header files and two source files main parameters that we would use them later to generate ,build ,validate and test the code generated.

## Simulink model Generator

We generate two Simulink models:

- System as whole (inputs ,filter ,controller ,plant, Outputs).

- RST Closed loop System only (REQUIRED MODULE).

## C-code Generator

We generate two C-codes:

- first well structured generated C code from the Automatic code generation in MATLAB.

- Second one for testing and validation and can be uploaded on the hardware and works very well as we used it for testing.

## Ide code generator(for testing)

This stage we use the generated a C-code form the previous  stage to simulate the closed loop system, that code generate an excel file with the value of Y(k) and U(K) (output and control signals)and plot them with time (k*Ts) to compare the code generated result with Simulink result.

- We initialization the  Gui using one of the Lecture Example as we set the values in the opening function.

Lecture example:

- Plant data :d=0 ,B(z)=.1 z-1+ .2 z-2 ,A (z)= 1 -1.3742 z-1 +48875 z-2.

- Tracking Dynamics: Bm(z)=.1 z-1+ .2 z-2 ,Am(z)= 1 -1.3742 z-1 +48875 z-2 , Ts=1 s,

ω=.5 rad/s ,ξ=.9.

- Regulation Dynamics P(z)=1 -1.3741z-1 +.4867z-2 ,Ts=1 s, ω=.4 rad/s ,ξ=.9 .

- After using bezout equation R(z)= 3 -3.94 z-1 +1.3141 z-2,S(z)= 1 - .3742 z-1 -.6258 z-2.

# Validation Method

- First we enter a lecture example into the Gui ,then click generate Definition files and then check configuration .h, configuration .c  ,RST Parameters .h , RST Parameters .c ,the header files should countian the definition of the vectors and the source should contain the values of the vectors.

- Then we click generate C-code ,it should generate  automatic C-code specific for Cortex M4  and Generic C-code for any system (using  the last header and source file generated in the previous step).

  - To validate and test the automatic code generated you can use:

    - SIL(software in the Loop) : as it convert the block into a C code or assembly code and simulate it on the software.

    - S function :it is a Block can convert the C files to MATLAB script and simulate it on the Simulink easily.

  - To validate the Generic code that I wrote to simulate the system:

    - First run the code by any C ide then it would generate an Excel file with the values of Y(K) ,U(k) and time (k*Ts).

    - Then we can select the columns and plot them on graphs and compare the result with the Gui result.

# Results (validation, Testing)

We test this example that I set in the Gui as a default.

1- click Generate Definition files  the four files

Figure 1:configuration.c file

Figure 2: configuration .h file

Figure 3:RST .c file

Figure 4: RST.h file

2-click Generate automatic C code files

The Automatic Code Generation would be generated for arm cortex-M4.

This code can be tested using 3 method

- Open ert_main.c and controller .c  files on any ide for simualtion Add the input step to and take the outputs U,Y values to an excel file and visualize them .

* Code generated for Simulink model 'System_20200611_1515_16_controller'.

*

* Model version                  : 1.0

* Simulink Coder version         : 8.14 (R2018a) 06-Feb-2018

* C/C++ source code generated on : Thu Jun 11 15:15:19 2020

*

* Target selection: ert.tlc

* Embedded hardware selection: NXP->Cortex-M4

* Code generation objectives: Unspecified

* Validation result: Not run

*/

#include <stddef.h>

#include <stdio.h>                     /* This ert_main.c example uses printf/fflush */

#include "System_20200611_1515_16_controller.h" /* Model's header file */

#include "rtwtypes.h"

/*

* Associating rt_OneStep with a real-time clock or interrupt service routine

* is what makes the generated code "real-time".  The function rt_OneStep is

* always associated with the base rate of the model.  Subrates are managed

* by the base rate from inside the generated code.  Enabling/disabling

* interrupts and floating point context switches are target specific.  This

* example code indicates where these should take place relative to executing

* the generated code step function.  Overrun behavior should be tailored to

* your application needs.  This example simply sets an error status in the

* real-time model and returns from rt_OneStep.

*/

void rt_OneStep(int Ts,FILE *fp,int k);

void rt_OneStep(int Ts,FILE *fp,int k)

{

static boolean_T OverrunFlag = false;

/* Disable interrupts here */

/* Check for overrun */

if (OverrunFlag) {

rtmSetErrorStatus(System_20200611_1515_16_cont_M, "Overrun");

return;

}

OverrunFlag = true;

/* Save FPU context here (if necessary) */

/* Re-enable timer or interrupt here */

/* Set model inputs here */

System_20200611_1515_16_contr_U.into=1;

if(k<100){

System_20200611_1515_16_contr_U.into2=0;

}

else{

System_20200611_1515_16_contr_U.into2=-.25;

}

/* Step the model */

System_20200611_1515_16_controller_step();

/* Get model outputs here */

if(k%1==0){

printf("%d ,%f ,%f\n",k,System_20200611_1515_16_contr_Y.Outform2 ,System_20200611_1515_16_contr_Y.Outform);

fprintf(fp, "%d,%d,%f,%f\n", k, k*Ts,System_20200611_1515_16_contr_Y.Outform2,System_20200611_1515_16_contr_Y.Outform);

}

/* Indicate task complete */

OverrunFlag = false;

/* Disable interrupts here */

/* Restore FPU context here (if necessary) */

/* Enable interrupts here */

}

/*

* The example "main" function illustrates what is required by your

* application code to initialize, execute, and terminate the generated code.

* Attaching rt_OneStep to a real-time clock is target specific.  This example

* illustrates how you do this relative to initializing the model.

*/

int_T main(int_T argc, const char *argv[])

{

/* Unused arguments */

(void)(argc);

(void)(argv);

/* Initialize model */

System_20200611_1515_16_controller_initialize();

/* Attach rt_OneStep to a timer or interrupt service routine with

* period 1.0 seconds (the model's base sample time) here.  The

* call syntax for rt_OneStep is

*

*  rt_OneStep();

*/

fflush((NULL));

FILE * fp = fopen("mydata.csv", "w");

if(fp == NULL){

printf("Couldnt open file");

return;

}

int Ts=1;

for (int i =0;i<300;i=i+1 ) {

rt_OneStep(Ts, fp ,i);

}

/* Disable rt_OneStep() here */

/* Terminate model */

System_20200611_1515_16_controller_terminate();

fclose(fp);

return 0;

}

/*

* File trailer for generated code.

*

* [EOF]

*/

/*

* The example "main" function illustrates what is required by your

* application code to initialize, execute, and terminate the generated code.

* Attaching rt_OneStep to a real-time clock is target specific.  This example

* illustrates how you do this relative to initializing the model.

*/

int_T main(int_T argc, const char *argv[])

{

/* Unused arguments */

(void)(argc);

(void)(argv);

/* Initialize model */

System_20200611_1515_16_controller_initialize();

/* Attach rt_OneStep to a timer or interrupt service routine with

* period 1.0 seconds (the model's base sample time) here.  The

* call syntax for rt_OneStep is

*

*  rt_OneStep();

*/

fflush((NULL));

FILE * fp = fopen("mydata.csv", "w");

if(fp == NULL){

printf("Couldnt open file");

return;

}

int Ts=1;

for (int i =0;i<300;i=i+1 ) {

rt_OneStep(Ts, fp ,i);

}

/* Disable rt_OneStep() here */

/* Terminate model */

System_20200611_1515_16_controller_terminate();

fclose(fp);

return 0;

}

/*

* File trailer for generated code.

*

* [EOF]

*/

The Result:

- Then generate the Sil module and build it in the module

The Result:

Figure 5: SIL module output

Figure 6: design output

Figure 7:compare between them

3- click generate Simulation C files

This file can run on code block or any ide to simulate the closed loop control system as this code generate an Exeal sheet with the control signal and the output signals.

# Conclusion

we were facing a problem which was modeling a controller for a specific plant was a complex problem then it was discovered the designing a controller using was possible but it still required lots of calculation and included a high percentage of error so it was better to let computers do the calculation ,so we design a computer program which take the plant Transfer function and the required performance parameters to generate the required transfer functions as well as the corresponding code which is specific for Arm Cortex-M4 with few adjustments.

# Video link on YouTube

https://www.youtube.com/watch?v=9USYxJDnOng&feature=youtu.be

# Appendix

GitHub link:

https://github.com/girgismicheal/Control-Project

%last edit 10/6/2020 by girgis micheal

%at the beginning we initialize the model with the lecture example as

%following

%at the beginning we initialize the model with the lecture example as following

% --- Executes just before RST_PP_v2 is made visible.

function RST_PP_v2_OpeningFcn(hObject, eventdata, handles, varargin)

% Choose default command line output for RST_PP_v2

handles.output = hObject;

% Update handles structure

guidata(hObject, handles);

set(handles.edit1, 'string','1');%Ts

%continuous plant

set(handles.edit2, 'string','0');%delay   %3 0

set(handles.edit3, 'string','1');%num

set(handles.edit4, 'string','[10 1]');%den

set(handles.checkbox1, 'value',0);%discrete plant

%discrete plant

set(handles.edit5, 'string','[0 0.1 0.2]');%Bp

set(handles.edit6, 'string','[1 -1.3 0.42]');%Ap

set(handles.checkbox2, 'value',0);%continuous design criteria

%regulation P(z)

set(handles.edit7, 'string','0.4');%w0

set(handles.edit8, 'string','0.9');%zeta

set(handles.edit11, 'string','[1 -1.3741 0.4867]');%P (just in case)

%tracking Bm(z)/Am(z)

set(handles.edit9, 'string','0.5');%w0

set(handles.edit10, 'string','0.9');%zeta

set(handles.edit12, 'string','[0.0927 0.0687]');%Bm

set(handles.edit13, 'string','[1 -1.2451 0.4066]');%Am (just in case)

set(handles.edit14, 'string','1');%Hr

set(handles.edit15, 'string','1');%Hs

set(handles.edit16, 'string','0');%disturbance cosine frequency

%set(handles.edit16, 'string','[1 -1]');%disturbance den

set(handles.checkbox3, 'value',0);

%set(handles.checkbox3, 'string','Unused');%cancel zeros

pushbutton2_Callback(0, 0, handles);

Gui initialization

% --- Executes on button press in pushbutton5.

function pushbutton5_Callback(hObject, eventdata, handles) %Generate Definition Files

% hObject    handle to pushbutton5 (see GCBO)

% eventdata  reserved - to be defined in a future version of MATLAB

% handles    structure with handles and user data (see GUIDATA)

[Ts Bp Ap Hr Hs dist P Bm Am]=acquire_data(handles);

%create file configration.h

fileP=fopen('configration.h','w');

fprintf(fileP,'#ifndef _CONFIG_H \n#define _CONFIG_H_\n');

fclose(fileP);

fileP=fopen('configration.h','a');

fprintf(fileP,'#define delay= %f ;', str2num(get(handles.edit2,'string')));

fprintf(fileP,'\n#define Ts=%f ;', Ts);

fprintf(fileP,'\nextern const float B[%d], A[%d], P[%d],Am[%d],Bm[%d];',numel(Bp),numel(Ap),numel(Ap),numel(Am),numel(Bm));

fprintf(fileP,'\n#endif');

fclose(fileP);

%create configration.c

fileP=fopen('configration.c','w');

fprintf(fileP, '#include \"configration.h\"\n');

fprintf(fileP, 'const float P[%d]=',numel(P));

fprintf(fileP, '{');

for k=1:numel(P)-1

fprintf(fileP, [num2str(P(k)) ',']);

end

fprintf(fileP, [num2str(P(k+1)) '};\n']);

%B(z)

fileP=fopen('configration.c','a');

fprintf(fileP, 'const float B[%d]=',numel(Bp));

fprintf(fileP, '{');

for k=1:numel(Bp)-1

fprintf(fileP, [num2str(Bp(k)) ',']);

end

fprintf(fileP, [num2str(Bp(k+1)) '};\n']);

%A(z)

fileP=fopen('configration.c','a');

fprintf(fileP, 'const float A[%d]=',numel(Ap));

fprintf(fileP, '{');

for k=1:numel(Ap)-1

fprintf(fileP, [num2str(Ap(k)) ',']);

end

fprintf(fileP, [num2str(Ap(k+1)) '};\n']);

fclose(fileP);

%Am(z)

fileP=fopen('configration.c','a');

fprintf(fileP, 'const float Am[%d]=',numel(Am));

fprintf(fileP, '{');

for k=1:numel(Am)-1

fprintf(fileP, [num2str(Am(k)) ',']);

end

fprintf(fileP, [num2str(Am(k+1)) '};\n']);

fclose(fileP);

%Bm(z)

fileP=fopen('configration.c','a');

fprintf(fileP, 'const float Bm[%d]=',numel(Bm));

fprintf(fileP, '{');

for k=1:numel(Bm)-1

fprintf(fileP, [num2str(Bm(k)) ',']);

end

fprintf(fileP, [num2str(Bm(k+1)) '};\n']);

fclose(fileP);

Bps=[]; Bpu=[];

if get(handles.checkbox3, 'value')  %cancel stable zeros

[Bps Bpu]=separate_B(Bp);

else                                %don't cancel

Bps=1; Bpu=Bp;

end

%SOLUTION

%Ap Hs S + Bp Hr R = P      (no zeros cancelled)

S_has_integrator=has_no_integrator(Ap);

Hs_full=[];

if S_has_integrator

Hs_full=conv(Hs, [1 -1]);

else

Hs_full=Hs;

end

A=conv(Ap, Hs_full)';

B=conv(Bpu, Hr)';

nA=size(A); nA=nA(1)-1;%vertical vector size, power of z = size-1

nB=size(B); nB=nB(1)-1;

A=padarray(A, nB-1, 0, 'post');

B=padarray(B, nA-1, 0, 'post');

M=[];

for k=1:nB

M=[M A];

A=circshift(A, 1);

end

for k=(nB+1):(nB+nA)

M=[M B];

B=circshift(B, 1);

end

%M

n=size(M);  n=n(1);%M is a square matrix

nP=size(P); nP=nP(2);

P_full=padarray(P', n-nP, 0, 'post');

SR=M\P_full;

%Extract R, S, T

R=SR((nB+1):(nB+nA))';

S=SR(1:nB)';

if S_has_integrator

S=conv(S, [1, -1]);

end

S=conv(S, Bps);

T=P/sum(Bpu);%(no zeros cancelled)

set(handles.edit17, 'string',['[' num2str(R) ']']);%full R S T values for Update plant & Simulink buttons

set(handles.edit18, 'string',['[' num2str(S) ']']);

set(handles.edit19, 'string',['[' num2str(T) ']']);

R=conv(R, Hr);

S=conv(S, Hs);

%create file RST.h

fileP=fopen('RST_parameters.h','w');

fprintf(fileP,'#ifndef _RST_H \n#define _RST_H_\n');

fclose(fileP);

fileP=fopen('RST_parameters.h','a');

fprintf(fileP,'extern const float R[%d], S[%d], T[%d];',numel(R),numel(S),numel(T));

fprintf(fileP,'\n#endif');

fclose(fileP);

%create RST.c

fileP=fopen('RST_parameters.c','w');

fprintf(fileP, '#include \"RST_parameters.h\"\n');

%R(z)

fprintf(fileP, 'const float R[%d]=',numel(R));

fprintf(fileP, '{');

for k=1:numel(R)-1

fprintf(fileP, [num2str(R(k)) ',']);

end

fprintf(fileP, [num2str(R(k+1)) '};\n']);

%S(z)

fileP=fopen('RST_parameters.c','a');

fprintf(fileP, 'const float S[%d]=',numel(S));

fprintf(fileP, '{');

for k=1:numel(S)-1

fprintf(fileP, [num2str(S(k)) ',']);

end

fprintf(fileP, [num2str(S(k+1)) '};\n']);

%T(z)

fileP=fopen('RST_parameters.c','a');

fprintf(fileP, 'const float T[%d]=',numel(T));

fprintf(fileP, '{');

for k=1:numel(T)-1

fprintf(fileP, [num2str(T(k)) ',']);

end

fprintf(fileP, [num2str(T(k+1)) '};\n']);

fclose(fileP);

Definition files Generator code

function pushbutton6_Callback(hObject, eventdata, handles)%Generate Simulation C Files

[Ts Bp Ap Hr Hs dist P Bm Am]=acquire_data(handles);

Bps=[]; Bpu=[];

if get(handles.checkbox3, 'value')  %cancel stable zeros

[Bps Bpu]=separate_B(Bp);

else                                %don't cancel

Bps=1; Bpu=Bp;

end

%SOLUTION

%Ap Hs S + Bp Hr R = P      (no zeros cancelled)

S_has_integrator=has_no_integrator(Ap);

Hs_full=[];

if S_has_integrator

Hs_full=conv(Hs, [1 -1]);

else

Hs_full=Hs;

end

A=conv(Ap, Hs_full)';

B=conv(Bpu, Hr)';

nA=size(A); nA=nA(1)-1;%vertical vector size, power of z = size-1

nB=size(B); nB=nB(1)-1;

A=padarray(A, nB-1, 0, 'post');

B=padarray(B, nA-1, 0, 'post');

M=[];

for k=1:nB

M=[M A];

A=circshift(A, 1);

end

for k=(nB+1):(nB+nA)

M=[M B];

B=circshift(B, 1);

end

%M

n=size(M);  n=n(1);%M is a square matrix

nP=size(P); nP=nP(2);

P_full=padarray(P', n-nP, 0, 'post');

SR=M\P_full;

%Extract R, S, T

R=SR((nB+1):(nB+nA))';

S=SR(1:nB)';

if S_has_integrator

S=conv(S, [1, -1]);

end

S=conv(S, Bps);

T=P/sum(Bpu);%(no zeros cancelled)

set(handles.edit17, 'string',['[' num2str(R) ']']);%full R S T values for Update plant & Simulink buttons

set(handles.edit18, 'string',['[' num2str(S) ']']);

set(handles.edit19, 'string',['[' num2str(T) ']']);

R=conv(R, Hr);

S=conv(S, Hs);

%TS and samples

fileP=fopen('main.c','w');

fprintf(fileP,['#include <stdio.h>\n#include <stdlib.h>\nint main()\n{\nFILE * fp = fopen("mydata.csv", "w");\nif(fp == NULL){\nprintf("Couldnt open file");\nreturn;\n}\n']);

fprintf(fileP,['float Ts=' num2str(Ts) ';\n']);

fprintf(fileP,['int n_samples=' num2str(1000) ';\n']);

fprintf(fileP,['float Ref[' num2str(numel(T)) ']' ';\n']);

fprintf(fileP,['float Uk[' num2str(numel(S)) ']' ';\n']);

fprintf(fileP,['float Ek[' num2str(numel(Bp)) ']' ';\n']);

fprintf(fileP,['float yk[' num2str(max(numel(conv(S,A)),numel(R))) ']' ';\n']);

Simulation  file Generator code

%to calculate y(k)

fileP=fopen('main.c','a');

NUm=conv(A,S)

BE=Bp/NUm(1)

NUm=NUm/-NUm(1)

fprintf(fileP, 'yk[0]=');

for k=1:numel(BE)

if(BE(k)>0)fprintf(fileP, ['+' num2str(BE(k)) '*Ek[' num2str(k-1) ']']);

else

fprintf(fileP, [num2str(BE(k)) '*Ek[' num2str(k-1) ']']);

end

end

for k=1:numel(NUm)-1

if(NUm(k+1)>0)fprintf(fileP, ['+' num2str(NUm(k+1)) '*yk[' num2str(k) ']' ] );

else fprintf(fileP, [num2str(NUm(k+1)) '*yk[' num2str(k) ']' ] );

end

end

fprintf(fileP,';\n');

fprintf(fileP,['\nif(k==1)yk[0]=.02*yk[0];\nif(k==100)yk[0]=yk[0]+.28;\n']);

fprintf(fileP,['printf(%cy[%c%c] = %c%c%c%c%c, k, yk[0]);\nfprintf(fp, %c%c%c,%c%c,%c%c,%c%c%c%c%c, k, k*Ts,-Uk[0],-yk[0]);\n'],'"','%','d','%','f','\n','"','"','%','d','%','f','%','f','%','f','\n','"');

fclose(fileP);

%shift old values

fileP=fopen('main.c','a');

fprintf(fileP,['for(int i=0;i<' num2str(numel(T)-1) ';i++)Ref[i+1]=Ref[i];\n']);

fprintf(fileP,['for(int i=0;i<' num2str(numel(S)-1) ';i++)Uk[i+1]=Uk[i];\n']);

fprintf(fileP,['for(int i=0;i<' num2str(numel(Bp)-1) ';i++)Ek[i+1]=Ek[i];\n']);

fprintf(fileP,['for(int i=0;i<' num2str(max(numel(conv(S,A)),numel(R))-1) ';i++)yk[i+1]=yk[i];\n}']);

fprintf(fileP,['fclose(fp);\nreturn 0;\n}']);

fclose(fileP);

%initailize the arrays

fileP=fopen('main.c','a');

fprintf(fileP,['for(int i=1;i<' num2str(numel(T)) ';i++)Ref[i]=0;\n']);

fprintf(fileP,['for(int i=1;i<' num2str(numel(S)) ';i++)Uk[i]=0;\n']);

fprintf(fileP,['for(int i=1;i<' num2str(numel(Bp)) ';i++)Ek[i]=0;\n']);

fprintf(fileP,['for(int i=1;i<' num2str(max(numel(conv(S,A)),numel(R))) ';i++)yk[i]=0 ;\n']);

fclose(fileP);

%simulation loop

fileP=fopen('main.c','a');

fprintf(fileP,['for(int k=0;k<n_samples;++k)\n{\n//simulate\n']);

fprintf(fileP,['Ref[0]=1;\n']);

%to calc e(k)

fileP=fopen('main.c','a');

fprintf(fileP, 'Ek[0]=');

for k=1:numel(T)

if(T(k)>0)fprintf(fileP, ['+' num2str(T(k)) '*Ref[' num2str(k-1) ']']);

else

fprintf(fileP, [num2str(T(k)) '*Ref[' num2str(k-1) ']']);

end

end

for k=1:numel(R)

if(R(k)>0)fprintf(fileP, ['+' num2str(R(k)) '*yk[' num2str(k-1) ']' ] );

else fprintf(fileP, [num2str(R(k)) '*yk[' num2str(k-1) ']' ] );

end

end

fprintf(fileP,';\n');

fclose(fileP);

%to calculate U(k)

fileP=fopen('main.c','a');

NUm=S/-S(1)

fprintf(fileP, 'Uk[0]=Ek[0]');

for k=1:numel(NUm)-1

if(NUm(k+1)>0)fprintf(fileP, ['+' num2str(NUm(k+1)) '*yk[' num2str(k) ']' ] );

else fprintf(fileP, [num2str(NUm(k+1)) '*yk[' num2str(k) ']' ] );

end

end

fprintf(fileP,';\n');

fclose(fileP);

Automatic code Generator  code

% --- Executes on button press in pushbutton4.

function pushbutton4_Callback(hObject, eventdata, handles)%Generate Automatic Code Files

% hObject    handle to pushbutton4 (see GCBO)

% eventdata  reserved - to be defined in a future version of MATLAB

% handles    structure with handles and user data (see GUIDATA)

% hObject    handle to pushbutton3 (see GCBO)

% eventdata  reserved - to be defined in a future version of MATLAB

% handles    structure with handles and user data (see GUIDATA)

[Ts Bp Ap Hr Hs dist P Bm Am]=acquire_data(handles);

R=str2num(get(handles.edit17, 'string'));%full R S T values

S=str2num(get(handles.edit18, 'string'));

T=str2num(get(handles.edit19, 'string'));

%global it_counter;

%it_counter=it_counter+1;

c=clock;

name=sprintf('System_%d%02d%02d_%02d%02d_%02d', c(1), c(2), c(3), c(4), c(5), round(c(6)));

nameofcont=sprintf('System_%d%02d%02d_%02d%02d_%02d_controller', c(1), c(2), c(3), c(4), c(5), round(c(6)));%controller

sim_time=100; t_ref=5; t_dist=50;

str_Ts=num2str(Ts);

x=0; y=0;

sys=new_system(name);

sys2=new_system(nameofcont);%controller

set_param(name, 'stoptime',num2str(sim_time));

set_param(nameofcont, 'stoptime',num2str(sim_time));%controller

x=x+0; y=y+100;%step position

add_block('simulink/Sources/Step', [name '/Reference'], 'position',[x, y, x+30, y+30],...

'time',num2str(t_ref*Ts), 'sampletime',str_Ts);

x=x+30;%step width

x=x+35;%spacing

add_block('simulink/Discrete/Discrete Filter', [nameofcont '/Bm(z)//Am(z)'], 'position',[x, y, x+195, y+30],...

'numerator',['[' num2str(Bm) ']'], 'denominator',['[' num2str(Am) ']'], 'sampletime',str_Ts);

x=x+195;%tracking width

x=x+35;%spacing

add_block('simulink/Discrete/Discrete Filter', [nameofcont '/T(z)'], 'position',[x, y, x+195, y+30],...

'numerator',['[' num2str(T) ']'], 'denominator','[1]', 'sampletime',str_Ts);

x=x+195;%T width

add_line(nameofcont, 'Bm(z)//Am(z)/1', 'T(z)/1');

x=x+25;%spacing

add_block('simulink/Math Operations/Sum', [nameofcont '/Sum1'], 'position',[x, y+5, x+20, y+5+20],...

'inputs','|+-');

x=x+20;%sum width

add_line(nameofcont, 'T(z)/1', 'Sum1/1');

x=x+20;%spacing

add_block('simulink/Discrete/Discrete Filter', [nameofcont '/1//Hs(z)'], 'position',[x, y, x+195, y+30],...

'numerator','[1]', 'denominator',['[' num2str(Hs) ']'], 'sampletime',str_Ts);

x=x+195;%Hs width

add_line(nameofcont, 'Sum1/1', '1//Hs(z)/1');

x=x+35;%spacing

add_block('simulink/Discrete/Discrete Filter', [nameofcont '/1//S(z)'], 'position',[x, y, x+195, y+30],...

'numerator','[1]', 'denominator',['[' num2str(S) ']'], 'sampletime',str_Ts);

x=x+195;%S width

add_line(nameofcont, '1//Hs(z)/1', '1//S(z)/1');

%add_line(name, 'Sum1/1', '1//S(z)/1');

x=x+330;%make space for possible continuous plant

add_block('simulink/Math Operations/Sum', [nameofcont '/Sum2'], 'position',[x+100+30, y+5, x+100+50, y+5+20],...

'inputs','++|');

x=x+20;%sum width

x=x-285;%return till after S

if get(handles.checkbox1, 'value')%continuous plant

add_block('simulink/Discrete/Zero-Order Hold', [name '/Zero-Order Hold'], 'position',[x-800, y+30, x+35-800, y+60]);

x=x+35;%ZOH width

%add_line(name, '1//S(z)/1', 'Zero-Order Hold/1');

x=x+35;%spacing

add_block('simulink/Continuous/Transport Delay', [name '/Plant Delay'], 'position',[x-800, y+30, x+30-800, y+60],...

'delaytime',get(handles.edit2, 'string'));

x=x+30;%delay width

add_line(name, 'Zero-Order Hold/1', 'Plant Delay/1');

x=x+35;%spacing

add_block('simulink/Continuous/Transfer Fcn', [name '/Gp(s)'], 'position',[x-800, y+30, x+130-800, y+80],...

'numerator',get(handles.edit3,'string'), 'denominator',get(handles.edit4,'string'));

x=x+120;%Gp width

add_line(name, 'Plant Delay/1', 'Gp(s)/1');

add_line(name, 'Gp(s)/1', 'Sum2/2');

else%discrete plant

add_block('simulink/Discrete/Discrete Filter', [nameofcont '/Bp(z)//Ap(z)'], 'position',[x+100, y+5, x+195+100, y+35],...

'numerator',['[' num2str(Bp) ']'], 'denominator',['[' num2str(Ap) ']'], 'sampletime',str_Ts);

x=x+195;%plant width

%add_line(name, '1//S(z)/1', 'Bp(z)//Ap(z)/1');

add_line(nameofcont, 'Bp(z)//Ap(z)/1', 'Sum2/2');

end

x=x+35;%spacing till sum2

x=x-135;%new branch

add_block('simulink/Sources/Sine Wave', [name '/Disturbance'], 'position',[x-1200, 170, x-1200+30, 200],...

'amplitude','-0.25', 'frequency',get(handles.edit16,'string'), 'phase','pi/2', 'sampletime',str_Ts);

x=x+30;%disturbance width

x=x+40;%spacing

add_block('simulink/Continuous/Transport Delay', [name '/Disturbance Delay'], 'position',[x-1200, 170, x-1200+30, 200],...

'delaytime',num2str(t_dist*Ts));

x=x+30;%delay width

add_line(name, 'Disturbance/1', 'Disturbance Delay/1');

x=x+125;%spacing from dist. delay till mux

add_block('simulink/Signal Routing/Mux', [name '/Mux'], 'position',[x-800, y, x-800+5, y+38]);

x=x+5;%mux width

x=x+35;%spacing

add_block('simulink/Sinks/Scope', [name '/Scope'], 'position',[x-800, y-5, x-800+30, y+42]);

%add_line(name, '1//S(z)/1', 'Mux/1');

add_line(name, 'Mux/1', 'Scope/1');

%add_line(name, 'Sum2/1', 'Scope/1');

%feedback blocks

x=x-465; y=y+110;%from scope left edge to R

block_R=add_block('simulink/Discrete/Discrete Filter', [nameofcont '/R(z)'], 'position',[x, y, x+195, y+30],...

'numerator',['[' num2str(R) ']'], 'denominator','[1]', 'sampletime',str_Ts, 'orientation','left');

%all=get(get_param(block_R, 'handle'))

x=x-195;%R width

%add_line(name, 'Sum2/1', 'R(z)/1');

x=x-35;%spacing

add_block('simulink/Discrete/Discrete Filter', [nameofcont '/Hr(z)'], 'position',[x, y, x+195, y+30],...

'numerator',['[' num2str(Hr) ']'], 'denominator','[1]', 'sampletime',str_Ts, 'orientation','left');

x=x-195;

add_line(nameofcont, 'R(z)/1', 'Hr(z)/1');

add_line(nameofcont, 'Hr(z)/1)', 'Sum1/2');

%add_line(name, 'R(z)/1)', 'Sum1/2');

add_block('simulink/Sinks/Out1', [nameofcont '/Outform'],'position',[1600, 100, 1620, 130])

add_block('simulink/Sinks/Out1', [nameofcont '/Outform2'],'position',[1600, 50, 1620, 80])

add_block('simulink/Sources/In1', [nameofcont '/into'],'position',[0, 100, 20, 130])

add_block('simulink/Sources/In1', [nameofcont '/into2'],'position',[1200, y-300, 1220, y-300+30])

add_line(nameofcont, '1//S(z)/1', 'Bp(z)//Ap(z)/1');

add_line(nameofcont, 'into/1', 'Bm(z)//Am(z)/1');

add_line(nameofcont, 'into2/1', 'Sum2/1');

add_line(nameofcont, 'Sum2/1', 'Outform/1');

add_line(nameofcont, 'Sum2/1', 'R(z)/1');

add_line(nameofcont, '1//S(z)/1', 'Outform2/1');

add_block('built-in/Subsystem', [name '/controller'],'position',[100, 85, 200, 200])

Simulink.BlockDiagram.copyContentsToSubsystem...

(nameofcont, [name +'/controller/'])

add_line(name, 'Disturbance Delay/1', 'controller/2');

add_line(name, 'Reference/1', 'controller/1','autorouting','smart');

%add_line(name, 'Sum2/1', 'controller/2','autorouting','smart');

add_line(name, 'controller/1', 'Mux/1');

if get(handles.checkbox1, 'value')%continuous plant

%add_line(name, 'controller/1', 'Zero-Order Hold/1','autorouting','smart');

else%discrete plant

%add_line(name, 'controller/1', 'Bp(z)//Ap(z)/1','autorouting','smart');

end

add_line(name, 'controller/2', 'Mux/2','autorouting','smart');

load_system(nameofcont)

cs = getActiveConfigSet(nameofcont);

cs.set_param('Solver', 'FixedStepauto');

switchTarget(cs,'ert.tlc',[])

set_param(nameofcont, 'ProdHWDeviceType', 'NXP->Cortex-M4')

rtwbuild(nameofcont)

open_system(sys);

open_system(sys2);

lineHandles = find_system(gcs,'FindAll','On','SearchDepth',1,'Type','Line');

# Contribution

Girgis Micheal: Searching and Understanding of the problem ,designing the solution ,Assign the team roles , Finding the Examples(Test cases), Testing and Validation Video , Software Finishing (calculate the modules).

Pierre Nabil: Code Generation Component.

John Bahaa: Testing and Validation.

Hazem Mousa: Definition Files Generation Component ,build on Keil.

Ibrahim Shoukry: Documentation ,Simulation code .

# One Drive Link

https://onedrive.live.com/?authkey=%21AJX5Rx4CKnc5U68&id=80BDBBE808EDCE73%21106&cid=80BDBBE808EDCE73

# Team members

Girgis Micheal

Pierre Nabil

Hazem Mousa

Ibrahim Shoukry

John Bahaa